Marco Thermos Machine De-scale Instructions

- Slide the black plastic filter pan into place (no filter paper required).

- Empty one sachet of 50g Renegite De-scaler into the 2.2 litre plastic jug of hot water and stir to ensure all the crystals have been fully dissolved.

- Decant the Renegite solution into the machine through the lid located on the top.

- Allow the machine to sit for 45 minutes for the de-scale solution to remove any traces of scale from within the internal boiler. Allow the water to cool as running the percolation process whilst hot can result in spitting and steam ejection. Ensure the decanter is placed below the black plastic filter pan before you turn the machine on.

- It is advisable to allow the de-scale solution to remain in the boiler overnight and empty it in the morning. Don’t forget to leave a note on the machine informing people not to use the machine whilst filled with descale solution.

- Allow the machine to run its full cycle (6-7 minutes) until the glass decanter has been filled with the descale solution.

- Fill the decanter with 12 cups of clean cold water and pour into the machine placing the jug below to ensure the water will be collected as the cycle is repeated. This flush through process will ensure the removal of remaining de-scale residue.

This preventative process will maintain the performance of your Marco Thermos machine and ensure its longevity.

This process should be undertaken on a regular basis as a preventative measure against the build- up of scale related material, regardless of your location and the hardness of your local water. This simple preventative process will maintain the performance of your Thermos Machine and ensure its longevity.

- Unless you live in an area of extremely soft water, a minimum of one sachet should be used every calendar month or every 50 sachets of coffee (the content of 1 standard Kingdom Coffee case of filter coffee).

Order code: 111-031 - Renegite 50g De-Scale Sachet